EDG Operator’s Handbook

Contents

Introduction	- 3 -

1.1 Features	- 3 -

1.2 Packing List	- 3 -

1.3 Charging the Battery	- 4 -

Operation of the EDG	- 4 -

2.1Power on/off	- 4 -

2.2 EDG MAIN	- 4 -

2.3 Model	- 5 -

2.3.1 New Curve	- 6 -

2.3.2 Model Settings	- 7 -

2.3.3 Delete a test point	- 10 -

2.3.4 Delete a curve	- 10 -

2.4 Project	- 10 -

2.4.1 New Project	- 11 -

2.4.2 Associate with a model	- 12 -

2.4.3 Start Test	- 13 -

2.4.5 Result View	- 15 -

2.4.5 Export the data	- 16 -

2.4.6 Delete a project	- 16 -

2.5 GPS Control	- 16 -

2.6 System Settings	- 17 -

Notice(Routine Operations)	- 17 -

Watch for suspicious readings	- 18 -

# Introduction

## 1.1 Features

EDG utilizes advanced electromagnetic technology to obtain accurate soil density readings.

Primary features:

※ No radioactive materials or special license required

※ Light weight and easy to use

※ Without compromising the integrity of the sample structure

※ Measures density in common units (g/cm³)

※ Measures for soil temperature

※ Stores 40000 readings on internal Data Logger

※ Optional download to computer

※ GPS, Bluetooth, WIFI and 4G inside

How does it work?

The density of soil is directly proportional to the measured dielectric constant of the material. EDG uses electrical waves to measure dielectric constant using an innovative, toroidal electrical sensing field established by the sensing plate.

The electronics in the EDG convert the field signals into material density readings and displays the results. Once calibrated, direct density readings can be consistently obtained.

As shown below:

## 1.2 Packing List

The EDG shipment package includes the items listed below. Please contact your dealer if any part missing.

- Instrument box

- One EDG unit

- One battery charger

- USB A-C cable

- User manual, warranty card and certificate

## 1.3 Charging the Battery

Note: Be sure to use the original charger for charging.

EDG is powered by lithium batteries which weigh less and smaller.

Under normal operation, EDG can operate in excess of 15 hours at full charge. The internal 3.7 volt battery is designed to be recharged in approximately 8 hours, using the charger with 5V output.

Warning！

Attempting to recharge the unit in any other way than with the recharger supplied with the unit can result in damage to the unit and can present a safety hazard. Use of any charging means other than the recharger supplied with the unit will void the unit's warranty.

# Operation of the EDG

Starting EDG for the first time. Turn EDG on by flipping the POWER switch on the left. The internal buzzer emits a beep sound. Then the unit begins self-test and starts. After a few seconds the display will show the MAIN menu.

## 2.1Power on/off

When the device is turned off, press the power button for 0.5 seconds and then release it. The instrument will start up for self-check and display the startup screen. Wait for a few seconds and then enter the MAIN MENU.

When the device is turned on, press and hold the power button for 4 seconds. The green working indicator light will remain on constantly. At this point, release the power button and the instrument will be turned off.

## 2.2 EDG MAIN

After boot up, the screen displays as follows:

Information

Battery Level

GPS State

Project Number

Model Number

Current Menu

The current menu interface, storage and status information are displayed at the top of the screen. When the EDG is located outdoors, the GPS status indicator icon is displayed; otherwise, it is not displayed.

## 2.3 Model

In the EDG MAIN menu, click the button "EDG START", the screen displays as follows:

Back

In the EDG START menu, click the button “MODEL”, turn to the MODEL menu:

### 2.3.1 New Curve

Click the button "New Model" and input a name:

### 2.3.2 Model Settings

In the MODEL menu, you can set the model by clicking the button "Settings". Menu as follows:

Press "New Point", Menu as follows:

Obtain readings at the positions as shown below.

Draw a circle around the sensor's outer edge of EDG and take readings in the center (1) and at 2 o'clock, 5 o'clock, 8 o'clock and 11 o’clock positions. After reading number 5 is complete, click “Save” to store the data.

In the MODEL SETTINGS menu, input the wet density and moisture by pressing "Modify". The wet density and moisture content are obtained through the sand-filling method or the ring knife method.

Warning!

Change the position to create at least three points. You should make sure the wet density and moisture of the three points are different.

After physical data of the three points has been inputted, you can fit this model by clicking “Fit”. Then a model is calibrated.

To calculate the compactness and moisture content, it is necessary to input the maximum dry density.

### 2.3.3 Delete a test point

In the MODEL SETTINGS menu, you can delete a point which is selected by pressing "Delete".

### 2.3.4 Delete a curve

In the "MODEL" menu, you can delete a model which is selected by clicking the button "Delete".

## 2.4 Project

In the EDG START menu ,Press the button “Project” , turn to the PROJECT menu.

### 2.4.1 New Project

In the PROJECT menu, you can create a project by clicking the button “New Project”.

Input a name of the new project, then click “Save”. You can see as follows:

Note: Up to 200 projects can be saved.

### 2.4.2 Associate with a model

A new project is not associated with any model. Click "Not Associated" area, it shows as follows:

Click the button "curve1", it will be associated with the current project.

### 2.4.3 Start Test

Select a project, and click the button "Start Test", test menu will be show:

EDG can be used in three different measurement or run modes, respectively:

Continuous Reading Mode

Single Reading Mode

Enhancement Mode

Continuous Reading Mode: Continuous reading mode is used to provide instantaneous density measurements for quality control purposes. When in continuous mode, EDG will display a screen and update values of density, temperature and compaction continuously.

Note: The data cannot be saved in this mode.

Single Reading Mode: single reading mode is used to provide density measurement for one specific point, and conduct single reading each time.

In the mode, click “Test” to start a reading. The unit begins to read and display data from the probe four seconds later after pressing “Test”.

Note: Any touch to the unit by hand or other metal objects is prohibited once the countdown of reading begins, otherwise will distort the reading seriously.

The data gained can be stored on internal data logger. Once a single reading is completed, press "Save" to save measured data.

Note: the unit allows maximum storage of data for 4200 points. You will be prompted to delete data, if exceed this number.

Enhancement Mode:

As shown below:

Draw a circle around the sensor’s outer edge of EDG and take readings in the center (1) and at 2 o'clock, 5 o'clock, 8 o'clock and 11 o’clock positions. After reading number 5 is complete, calculate the average of the 5 readings. Press "Save" to store the data after the measurement is completed.

### 2.4.5 Result View

In the PROJECT menu, click the project name. You will enter the menu which you can view results and delete test points.

### 2.4.5 Export the data

In the RESULT VIEW menu, you can convert the data into an Excel file named for this project by clicking “Make Report”. After the EDG is connected to your computer by a USB cable, you can find a new disk. The report is in the EXCEL folder on the new disk. Then you can check it on your computer.

### 2.4.6 Delete a project

In the PROJECT menu, you can delete one project which is selected by clicking the button "Delete".

## 2.5 GPS Control

In the MAIN MENU, you can turn on/off the GPS by clicking the button “GPS CONTROL”.

## 2.6 System Settings

In this menu, you can set language、time and backlight. In addition, you can also check for the firmware updates.

# Notice(Routine Operations)

1).Once calibrated the EDG is ready to be used at each job site and asphalt mix. It is suggested that 5 readings be taken at each site following the average reading pattern.

2).Keep EDG bottom surface clean and dry. For accurate readings, the EDG should have a clean, dry and smooth interface with the mat. Therefore, you should wipe the EDG probe surface dry and clean after each reading.

3).Before placing the EDG on the mat, you should check the surface of the mat to make sure there is no loose material on the surface. If a buildup of asphalt begins to form on the probe surface, clean the surface with WD-40.

4).Choose a flat measuring point. While the EDG contains moisture correction algorithms, the most accurate readings will be obtained if areas with low moisture levels are measured. If a measurement area has noticeable surface moisture, you should either wait for the moisture to evaporate or remove the moisture with an absorbent cloth.

# Watch for suspicious readings

If a reading seems unusual or suspicious, check for possible measurement errors (dirty surface, wet surface, low battery voltage) and retake the reading.

Rest results are obtained when moisture levels for a series of readings are kept relatively constant. If the moisture level for a reading is more than a percentage point higher or lower than the previous readings, the reading should be treated as suspicious.

Don't touch the EDG while it is making a measurement. Touching any part of the EDG while it is performing a measurement can distort the reading.